To Install the Lava Link-650 card in an NT system:

1)	Configure the Link-650 card for the desired Port # and a free IRQ.

Eg: COM3:, IRQ 10

2)	Boot NT, logging on as "administrator"

3)	Access the Lava Drivers diskette and switch to the WinNT4.099 sub directory.

4)	Before proceeding with the next step, ensure that your NT system is installed in the default directories, ie WINNT\SYSTEM32.  If your system uses a different sub directory you will have to manually copy all files.  See below.

Execute install.bat.  This file will

a: copy the driver SerL650.sys into the WINNT\SYSTEM32\DRIVERS sub directory,

b: copy the control panel applet LINK650.CPL file into WINNT\SYSTEM32

c: start the control panel applet.

Note: if your system sub directories are not the default directories as described above, copy the above mentioned files manually into the appropriate directories.  Then open Control Panel and execute the new Lava Link-650 Ports Applet.

5)	The following screen will appear (depending on your installed serial ports, the listing under ports may vary)

6)	Select the Com Port whick corresponds to the newly installed Link-650 card, (in our example 	COM3) and click  Settings.

7)	The following screen, (entries may differ) will appear.  Select Advanced.

8)	The following screen will appear ( settings will differ)

your COM Port Number should correspond to the COM port jumper setting on the Lava Link 650

Leave the Base I/O Port Address set to default, it will automatically select the proper address 	setting.

Select the IRQ setting whick corresponds to your hardware jumper setting on the Link-650 card.

Select Link-650 as the Uart Type.

Select OK

9)	The following dialog box will appear.

Select Restart Now, and your system will reboot.

The Link-650 is now settup for operation.

Note: The Lava Link-650 Control Pannel applet, can be used to configure all your Serial Ports under Windows NT.  It is recommended that this applet be used for future configurations of all ports, as it does provide extra features not available with the standard Ports applet.  For total compatability with NT, do not remove the original Ports applet from your system; the two coexist without any interference.  It is however not possible to configure the Link-650 through the standard Ports applet.